Fire / Rescue Response

COMPLETIONS

“Fracing”

STAGE

BLOWOUT or WELL CONTROL INCIDENT

(WITH FIRE)

BLOWOUT or WELL CONTROL INCIDENT

(w/o Fire)

INJURY ON SITE OR

OTHER INCIDENT

1. Contact Tool Pusher and Drilling Consultant

2. Determine if wells production is classified as  Non-H2S or H2S

If Non-H2S production utilize Emergency Response Guidebook Guide 115 (Natural Gas) to initiate response actions.

If H2S production utilize Emergency response Guidebook Guide 117 (Hydrogen Sulfide) to initiate response  actions

If the production classification is unknown utilize Emergency Response Guide Book Guide 117 (Hydrogen Sulfide) to initiate response actions until the situation can be further evaluated and determined.

1. Contact Site Supervisor

2. Assess incident scenario (e.g. confined space, rescue at heights, etc) to determine appropriate response actions.

Assess with what steps need to be taken to secure site for safety of responders (e.g., site shut down, electrical disconnects, etc)

1. Assess injury type

2. If chemicals involved, have site crew

advise of type

3. Secure chemicals from area

4. Acquire MSDS for chemical

Begin Traffic Control

Site Crew to make area safe for access to injured, then begin rescue of injured

Transport Injured

Notify Governmental Agencies if needed

Establish Site Security

Identify Hot, Warm, Cold Zones

Establish Air Quality Sampling

If fire is deemed controllable by Toolpusher   / Consultant, responders may then fight fire

If fire is uncontrollable – Wait on Well Control Specialists to arrive – Protect Collateral Assets

Respond & Treat Injured – If H2S gas decontaminate injured and notify medical personnel to prevent exposure or injuries to them.

Develop liaison with all Governmental Agencies required.

1. Contact Tool Pusher and Drilling Consultant

2. Determine if wells production is classified as Non-H2S or H2S

If Non-H2S production utilize Emergency Response Guidebook Guide 115 (Natural Gas) to initiate response actions.

If H2S production utilize Emergency response Guidebook Guide 117 (Hydrogen Sulfide) to initiate response actions.

If the production classification is unknown utilize Emergency Response Guide Book Guide 117 (Hydrogen Sulfide) to initiate response actions until the situation can be further evaluated/determined.

Establish Site Security

Identify Hot, Warm, Cold Zones

Establish Air Quality Sampling

Contain fluids by diking outside immediate area & insuring none can reach any ditches or streams.

If well is uncontrolled wait on Well Control Specialists to arrive.- Protect Collateral Assets

Dispatch spill control response team & monitor control of spill until response team arrives.

Develop liaison with all Governmental Agencies required.

A release from something other than the Well

(e.g., tanks, pits, etc)

1. Contact Tool Pusher and Drilling Consultant

2. Determine source of release (tank, pit, drums, etc).

3. Utilize Emergency Response Guide book to initiate response actions based on material being released.

Establish Site Security

Identify Hot, Warm, Cold Zones

Establish Air Quality Sampling

Contain fluids by diking outside immediate area & insuring none can reach any ditches or streams.

Dispatch spill control response team & monitor control of spill until response team arrives.

Develop liaison with all Governmental Agencies required.

Lycoming County Gas Exploration Taskforce Safety Committee                                              7/31/2026