Fire/Rescue Response

DRILLING

STAGE

BLOWOUT or WELL CONTROL INCIDENT

(WITH FIRE)

BLOWOUT or WELL CONTROL INCIDENT

(w/o Fire)

INJURY ON RIG OR

OTHER INCIDENT

1. Contact Tool Pusher and Drilling Consultant

2. Determine if wells production is classified as Non-H2S or H2S

If Non-H2S production utilize Emergency Response Guidebook Guide 115 (Natural Gas) to initiate response actions.

If H2S production utilize Emergency response Guidebook Guide 117 (Hydrogen Sulfide) to initiate response. actions

If the production classification is unknown utilize Emergency Response Guide Book Guide 117 (Hydrogen Sulfide) to initiate response actions until the situation can be further evaluated/determined.

1. Contact Tool Pusher and Drilling Consultant.

2. Assess incident scenario (e.g. confined space, rescue at heights, etc) to determine appropriate response actions.

Assess with what steps need to be taken to secure site for safety of responders (e.g., rig shut down, electrical disconnects, etc)

1. Assess injury type

2. If chemicals involved, have rig crew

advise of type.

3. Secure chemicals from area

4. Acquire MSDS for chemical

Begin Traffic Control

Rig Crew to make area safe for access to injured, then begin rescue of injured.

Transport Injured

Notify Governmental Agencies if needed

Establish Site Security, Identify Hot, Warm, Cold Zones, Establish Air Quality Sampling

If fire is deemed controllable by Toolpusher/Consultant, responders may then fight fire.

If fire is uncontrollable – Wait on Well Control Specialists to arrive – Protect Collateral Assets

Respond & Treat Injured – If H2S gas decontaminate injured and notify medical personnel to prevent exposure or injuries to them.

Develop liaison with all Governmental Agencies required.

1. Contact Tool Pusher and Drilling Consultant

2. Determine if wells production is classified as Non-H2S or H2S

If Non-H2S production utilize Emergency Response Guidebook Guide 115 (Natural Gas) to initiate response actions.

If H2S production utilize Emergency response Guidebook Guide 117 (Hydrogen Sulfide) to initiate response actions.

If the production classification is unknown utilize Emergency Response Guide Book Guide 117 (Hydrogen Sulfide) to initiate response actions until the situation can be further evaluated/determined.

Establish Site Security, Identify Hot, Warm, Cold Zones

Establish Air Quality Sampling

Contain fluids by diking outside immediate area & insuring none can reach any ditches or streams.

If well is uncontrolled wait on Well Control Specialists to arrive.   Protect Collateral Assets.

Dispatch spill control response team & monitor control of spill until response team arrives.

Develop liaison with all Governmental Agencies required.

A release from something other than the Well             (e.g., tanks, pits, etc)

1. Contact Tool Pusher and Drilling Consultant

2. Determine source of release (tank, pit, drums, etc).

3. Utilize Emergency Response Guide book to initiate response actions based on material being released.

Establish Site Security, Identify Hot, Warm, Cold Zones, Establish Air Quality Sampling

Contain fluids by diking outside immediate area & insuring none can reach any ditches or streams

Dispatch spill control response team & monitor control of spill until response team arrives.

Develop liaison with all Governmental Agencies required.