WELL CONTROL RESPONSE CONSIDERATIONS

What Constitutes a Well Control Emergency?

- Uncontrolled flow from the wellbore

- During Drilling Operations

- During Workover Operations

- During Production or Hydraulic Fracturing Operations

- During interface with antiquated or orphaned surface equipment

- The flow could be oil, gas, condensate, brine, sand, gravel, rock, steam or any combination...

Guidance for dispatchers:

- Ask These Important Questions:

- Is the rig on fire?

- Is there a visible gas cloud present?

- Is there any unusual noises emanating from the rig?

- Has anyone been injured or evacuated from the location

Critical steps for first on-scene:

DO's

- Protect population.

- Protect collateral assets.

- Establish remote air monitoring.

- Wait on Well Control Specialists to arrive.

DONT's

- Attempt to extinguish fire.

- Attempt to conduct victim recovery on well/rig.

- Attempt to mitigate leaks by randomly closing valves or other means.

- Attempt to move vehicles away from well/rig site.

Primary Safety Considerations/Priorities:

- Assess potential for well control event to escalate

- Burning?

- Gas cloud?

- Establish a conservative hot zone.

- Evacuation of populations at risk.

- Initiate remote air monitoring

- Implement County Emergency Response Plan.

- If safe to do so, implement pollution abatement measures external to the warm zone.

- Size up the incident.

- Evacuate residents at risk.

- Quickly organize response operations and set-up a command post.

- Implement tactics to achieve site control, site safety, and protect responders, the environment, and adjoining property.

- Identify the responsible party.

- Get appropriate mutual aid and well control expertise enroute.

Secondary hazards on site:

- Explosive Fumes & Toxic Gas

- Ignition Sources

- Fuel Tanks

- Toxic Chemicals (Pits)

- Explosive / Radioactive Logging Tools

- Flammable Liquids

- Compressed Gases

- Explosive Fumes & Toxic Gas

- Ignition Sources

- Fuel Tanks

- Toxic Chemicals (Pits)

- Explosive / Radioactive Logging Tools

- Flammable Liquids

- Compressed Gases

Hazmat Op:

Control of Migrating Liquids

& Vapors

- Ditching

- Damming

- Diking

- Booms & Sorbents

- Vapor Suppression

- Foam

- Water knock down

Initial Safety Management:

- Establish conservative work zones

- Hot Zone = 0' To 250' around wellhead

- Warm Zone = 250' to 500' around wellhead

- Cold Zone = 500' to 1000' around wellhead

- Set up atmospheric monitoring.

- Arrange for Emergency Medical Services and establish Medivac L.Z (if applicable)

- Conduct evacuations of people and resources at risk

Suburban Interface Considerations:

- Air Pollution

- H2S

- Gas Plume - Explosive Concentration?

- Smoke

- Ground Pollution

- Contamination of Water Supply

- Contamination of Ditches / Public Drain Systems

- Ignition Forces

- Secondary Explosions

- Heat Radiation

- Broaching of Surface

- Utility damage

- Power Lines

- Pipelines

- Area Population

- Area Evacuation

- Transportation

- Housing & Catering

- Water Sourcing

- Storage

- Adjacent Land Availability

- Tight location

- Equipment Staging Area

- Alternate Access Route

- Multiple Well Pads

- Media Coverage

- Over flights (establishing a "No Fly" zone)

Summary points/questions:

- What constitutes command and control on a well control emergency?

- What type of air monitoring will be required?

- Who has authority to request well control and firefighting expertise?

- How much water is needed to commence operations?

- How long will municipal responders be expected to stay?

- What stakeholders are likely to be on scene?

- An organized team/unified command approach in responding to an oilfield emergency keeps everyone safe, informed and "in-the-loop."

- Cautious size-ups are mandatory to determine if external resources will be required.

- A conservative Hot Zone is preferential during the early phases of a well control emergency.

- Continuous risk assessment is a major component of managing safety at a well control emergency.

- The transparent communication of mitigating actions underway is critical to stakeholder understanding and support during a well control emergency.

- Who is responsible for site security?

- Who manages off-site pollution?

- Should we hold a town hall meeting about the incident?

- How far away should our concentric evacuation zones be?

- What does the hospital ER need to know?

- Where do we put the medivac LZ? (if applicable)

- What type of hose connection do well control specialists use?

- What do we need to wear for PPE?

- How large is the "no-fly" zone? -