3.0     FREQUENCY LIST

3.1	Currently, there are twenty frequencies licensed for fire/rescue/EMS Communications in Lycoming County and two in Sullivan County. All mobile and portable communications utilizing these channels shall adhere to procedures as set forth by the Lycoming County Communications Center in conjunction with the Lycoming County Fire Chief’s advisory board and Part 90 of the Federal Communications Commission (FCC).  These frequencies and their primary use are listed below:

FREQUENCY ONE - LYCOMING COUNTY FIRE (158.8500 Repeater Input and 154.4300 Repeater Output MHz) Primary dispatch frequency for all Lycoming County Fire/Rescue/EMS/Fire Police services. (VHF Private Line Frequency for Lycoming County is 167.9 Hz.)

FREQUENCY TWO - (Tactical Frequency) (153.9500 MHz)

For tactical fire-ground and rescue communications.

FREQUENCY THREE (Tactical Frequency) (154.0100 MHz) –

For tactical fire-ground and rescue operations.

FREQUENCY FOUR (Tactical Frequency) (155.3400 MHz) –

For EMS operations only.

FREQUENCY FIVE (Tactical Frequency) (154.3400 MHz) –

For tactical fire-ground and rescue operations.

FREQUENCY SIX (Tactical Frequency) (154.4000 MHz). –

For tactical fire-ground and rescue communications.

COUNTYWIDE FREQUENCY (151.3175/154.8000)

This frequency is for interagency communications between police, fire, EMS, and EMA when multiple public safety agencies are involved in a large scale incident.  Examples are: search and rescues – lost hunter, mass casualty incidents, airport incidents, large scale police incidents.  It can also  be used for priority messages for fire, police, EMS, and EMA where tactical frequencies do not provide direct radio capabilities, such as an EMS unit talking to the police unit or an EMA unit talking to a fire unit, etc.    Lycoming County police agencies will be using this frequency for priority incidents where multiple police agencies are involved and as a traffic channel when staffing permits.

The Lycoming County Communications Center will have direct authority over this frequency, which operates off of a repeater. The repeater will routinely be turned off.   If a unit/department needs to use this frequency they must contact the Communication Center, give the purpose of the use and obtain permission.  Once permission is granted, the Telecommunicator will turn the repeater on for that specific incident.  Once the incident is terminated the repeater will be turned off.  If the frequency is in use and a second request comes in the Shift Supervisor will need to make the decision on which incident has priority.  Keep in mind that all police units now have EMA and it could be used in place of countywide for a large police incident.

LOCAL MUNICIPAL FREQUENCY

DESIGNATED FIRE POLICE - (155.9850 Mhz).

Used for fire police operations and coordination with other fire police agencies.

LYCOMING COUNTY FIRE SIMPLEX Channel (154.4300 MHz)

SULLIVAN COUNTY

FREQUENCY ONE - SULLIVAN COUNTY FIRE (154.1300 MHz)

Primary dispatch frequency for all Sullivan County fire/rescue/EMS services.  Currently Sullivan County is running a VHF Highband system.

FREQUENCY TWO –(154.0700) SULLIVAN COUNTY FIRE  LAPORTE

FREQUENCY THREE - (Tactical Frequency) (154.0700 MHz)

Tactical fire-ground communications. NORTH MNT

FREQUENCY FOUR – (154.0700) NORTH MNT

FREQUENCY 5 – (154.1300) NOT FOR FIRE GROUND USE

FREQUENCY 6 – (154.0700) COMMAND & LZ

FREQUENCY 7 – (154.3025) OPS

FREQUENCY 8 – (154.3475) WATER SUPPLY\

FREQUENCY 9 – (155.9925) FIRE POLICE

FREQUENCY 10 –  (158.9325) EMS/RIT/OTHER

3.2	MED FREQUENCIES

The following frequencies are UHF special emergency channels authorized for use ONLY by EMS units (ambulance/BLS or ALS):

MED One - 463.000 and 468.000
MED Two - 463.025 and 468.025 (Regional Back-Up Calling Channel)
MED Three - 463.050 and 468.050
MED Four - 463.075 and 468.075 (Statewide Calling Channel)
MED Five - 463.100 and 468.100
MED Six - 463.125 and 468.125
MED Seven - 463.150 and 468.150
MED Eight - 463.175 and 468.175
MED Nine - 462.950 and 467.950 (Dispatch and Coordination)
MED Ten - 462.975 and 467.975 (Used for coordination between field unit, hospital to hospital, or field unit to hospital)